Homework 7

Due 22 April 2004

Lossless media

- The magnetic field of a wave propagating through a certain non-magnetic material is given by

[mA/m]

Find;

- The direction of wave propagation

- The phase velocity, up

- The wavelength in the material, λ

- The relative permittivity of the material,  εr

- The electric field phasor,

- The electric field in time domain form,

Average Power Density

2. The free space safety standard of electromagnetic radiation (i.e. the level of radiation should not be exceeded to avoid any hazardous effects) is 10 [mW/cm2]. Express this safety standard in terms of average value of electric and magnetic fields of a plane wave in free space.

Lossy media

- Find the complex propagation constant γ and the intrinsic impedance η of a microwave signal in muscle tissue at 915 MHz. (σ=1.6 S/m, εr=51).

Polarization

- For a wave characterized by the electric field

,

determine the polarization state, determine the polarization angles (γ, χ), and sketch the locus of  for each of the following cases:

- A=3[V/m], B=4 [V/m], and δ=180°

- A=3[V/m], B=3 [V/m], and δ=45°

- A=3[V/m], B=4 [V/m], and δ=-135°