PREPARATION ASSIGNMENTS-7

Due Thursday, April 15

The electric field phasor of a uniform plane wave traveling in a lossless medium with an intrinsic impedance 0f 188.5 [Ω] is given by  [mV/m]. Determine

- The associated magnetic field phasor

- The instantaneous expression for if the medium is non-magnetic (μ=μ0)

Due Monday, April 19

For each of the following combinations of parameters, determine if the material is a low-loss dielectric, a quasi-conductor, or a good conductor and then calculate α, β, λ, up, ηc

- Glass with μr=1, εr=5 and σ=10-12 [S/m] at 10 GHz

- Wood with μr=1, εr=3 and σ=10-4 [S/m] at 1 kHz

Due Wednesday, April 21

Traveling and waves in free space (region 1) are normally incident on the interface with a perfect dielectric (region 2) for which εr=3. Find the reflection and transmission coefficients forfor a lossless media.

Due Monday, April 26

A parallel polarized plane wave is incident from air onto a dielectric medium with εr=9 at the Brewster angle. What is the refraction angle?